Part 1.

1. The process of Marr-Hildreth Detector (LoG) is:

(1) Smooth the image by applying a Gaussian filter on the image, parametrized by the window size n. Since 2nd order derivatives are quite sensitive to noise.

Gx, y= e-x2+y22σ2

(2) Use 2nd order derivative to detect the edges. So calculate the Laplacian of the output Gaussian from step 1.

2Gx, y= [x2+y2-2σ2σ4]e-x2+y22σ2

The output would be:  gx, y= 2Gx, y f(x, y)

(3) Since the Laplacian output (2nd order derivative) generates negative signs of the neighbors for every edge, so we need to find the zero crossing in the output from step 2 to locate every edge.

The size of the Gaussian window n is typically chosen as an odd number that is >= 6 * σ.

The process of a Canny Edge Detector is:

(1) Smooth the image by applying a Gaussian filter, in order to reduce the impact of noise.

(2) Compute the partial derivatives as gradient, and use approximations to compute the gradient magnitude.

gradf= gxgy=∂f∂x∂f∂y, edge strength Mx, y= gx2+gy2 ≈ gx+ gy

(3) Apply non-maxima suppression on the gradient magnitude to thin the edges to single edge point. The basic idea here is to explore in the direction of gradients, and preserve the largest edge strength value as edge, and suppress all other values.

(4) Detect and refine edges by conducting double thresholding, steps are:

a) plot the edge values of each pixel number along gradient maxima generated from step 3.

b) set 2 thresholds: high threshold t1 and low threshold t2.

c) edge values higher than t1 are determined as edge.

d) edges connected to strong edge pixels are determined as edge.

e) all other pixels are not considered as edge.

2. The polar representation of a line:

ρ=xcosθ+ysinθ

in a plane of an image, the range of θ and ρ: θ ∈-π2, π2, ρ∈[-2n, 2n], where the image is in the size of n × n.

The process of Hough Transform to locate lines:

1) Cut and partition the image plane into accumulator cells A[ρ, θ], so that the cell at (i, j) represents the square associated with parameter values (θi, ρi).

2) Initialize all cells with value 0.

3) For each point in (xk, yk) in the image:

3.1) calculate all ρq based on ρ = xcosθ + ysinθj, where θj are all possible values partitioned in step 1.

3.2) round these ρq to its closest cell value

3.3) increment the cell value of A(q, j)

4) Now, every cell in matrix A contains the number of points in the (x, y) space that lie on the same line.

5) find the line candidates where A(i, j) is beyond a certain threshold.

3.

1) Load an image:

I = imread('ku.jpeg');

imshow(I);

2) Detect edges using Canny detector with thresholds of [0.2, 0.5]:

grey = rgb2gray(I);

BW = edge(grey, 'canny', [0.2 0.5]);

imshow(BW);

3) Try with different threshold combinations:

i = 0;

for a=[0.1, 0.3, 0.4, 0.6]

bw = edge(grey, 'canny', [a a+0.3]);

i = i + 1;

subplot(2,2,i),imshow(bw); title(strcat('thresholds=[', num2str(a), ', ', num2str(a+0.3), ']'));

end

With the value of thresholds going up, it tends to detect edges that are more essential, and ignoring more subtle edges.

4) construct Hough peaks and compute Hough lines based on it.

P = houghpeaks(H, 5, 'threshold', ceil(0.3*max(H(:))));

lines = houghlines(BW, theta, rho, P, 'FillGap', 5, 'MinLength', 7);

Traverse through all lines detected by houghlines, plot 2 primary lines: the first one, and the longest one.

figure, imshow(grey), hold on

max_len = 0;

for k = 1:length(lines)

xy = [lines(k).point1; lines(k).point2];

if k == 1

plot(xy(:,1),xy(:,2),'LineWidth',2,'Color','green');

% Plot beginnings and ends of lines

plot(xy(1,1),xy(1,2),'x','LineWidth',2,'Color','yellow');

plot(xy(2,1),xy(2,2),'x','LineWidth',2,'Color','red');

end

% Determine the endpoints of the longest line segment

len = norm(lines(k).point1 - lines(k).point2);

if ( len > max_len)

max_len = len;

xy_long = xy;

end

end

% highlight the longest line segment

plot(xy_long(1,1),xy_long(1,2),'x','LineWidth',2,'Color','yellow');

plot(xy_long(2,1),xy_long(2,2),'x','LineWidth',2,'Color','red');

plot(xy_long(:,1),xy_long(:,2),'LineWidth',2,'Color','red');

4. The process of finding the least-square solution of a linear system Ac = y:

1) define the residual error between the predicted value and real value:

ri= yi-kxi+b, minimizeri2

2) Formulate the cost function:

J= i=1mri2= r2=y-AcTy -ac= yTy-2yTAc+cTATAc

3) Take the partial derivative on c and make it equal to 0 to find minimum:

∂J∂c= -2yTA+2ATAc=0

ATAc= ATy, c=(ATA)-1ATy

4) compute c according to the formula, and the final solution would be y = c1x + c2

For the below data points:

y = [1 1.5 1 2 2.5]';

A = [1 1; 2 1; 3 1; 4 1; 5 1 ];

c = inv(A'*A)*A'*y

c =

0.3500

0.5500

y = 0.35x + 0.55

5.

1) To compute the first order derivative for gradient, we calculate the partial derivative of the cost function on our target c: ∂J∂c, and find the minimum point by checking when it equals to zero.

For edge strength, we calculate the gradient of pixel values in both x axis and y axis, by computing the partial derivative on x and on y: gradf= gxgy=∂f∂x∂f∂y. The strength of the edge can be represented by an approximation in magnitude: x, y= gx2+gy2 ≈ gx+ gy.

For edge direction, the direction angle could be represented using arctan:

αx, y= tan-1gygx

2) The masks for Prewitt, Sobel and Laplacian detectors:

| 0101-41010 1111-81111 |  | -101-101-101 -1-10-101011 |  | -101-202-101 -2-10-101012 |  |
| --- | --- | --- | --- | --- | --- |
| normal | diagonal | vertical | diagonal | vertical | diagonal |
| Laplacian |  | Prewitt |  | Sobel |  |

Laplacian: use second order derivative for edge detection, with 3 × 3 filter.

lab=[0 1 0;1 -4 1; 0 1 0];

rez=uint8(filter2(lab grey, 'same'));

imshow(rez);

title('laplacian');

Prewitt: use first order derivative for edge detection, with 3 × 3 filter.

E2 = edge(grey, 'prewitt');

figure; imshow(E2); title('prewitt');

Sobel: use first order derivative for edge detection, with smoothing added.

E3 = edge(grey, 'sobel');

figure; imshow(E3); title('sobel');

Part 2.

1. A brief technical description of the blind deconvolution algorithm

Blind deconvolution  is a technique to recover a sharp version of an image from a blurred input, denoted by y=k x, where k is an unknown blur kernel.

The paper explains about the failure of the MAPx,k (maximum-a-posteriori) estimation approach, which seeks a (x, k) pair that maximizes px, ky∝pyx, kpxp(k). A possible reason of failure could be the choice of a bad prior in natural image scenarios, which fails to recover the true blur kernel.

To overcome the limitations of MAPx,k approach, the paper discussed about reconsidering the choice of estimator instead of the choice of prior, which is the MAPK estimation approach. Since the size of the kernel k is fixed and small compared to x, thus to exploit asymmetry between x and k would be wise, and this is why it selects the k that maximizes pky denoted by pky= py kpk/p(y), and x is marginalized, whose full possible values are evaluated using integral.

The paper elaborates two MAPK estimations: Gaussian prior and sparse prior. The Gaussian prior is in the form of X ~ N(0, diag(σω2))-1 where σω2= β(Gx,ω2+Gy,ω2)-1, Gx and Gy denote the Fourier transform of derivatives gx, gy. In this way, the solution for MAPK would result from the prior variance σ2 and looks like a sample from the prior, instead of a X=0 solution as  MAPx,k did.

For a sparse prior, several approximation strategies could be applied for computation simplicity to replace a closed form solution, such as the histogram approximation that sums over all gradient histogram bins instead of all image pixels, and variational Bayes approximation that assumes px, ky follows a Bayesian distribution and tries to solve x using a picked blur kernel from the distribution.

2. Description on each step.

(1) load image and transform to grayscale.

I = imread('ku.jpeg');

grey = rgb2gray(I);

imshow(grey);

(2) blur the image using a Gaussian filter.

PSF = fspecial('gaussian', 7, 10);

Blurred = imfilter(grey,PSF, 'symmetric', 'conv');

imshow(Blurred)

title('Blurred Image')

(3) Try to restore the blurred image using point-spread function. First try UNDERPSF in a uniform array with 4 pixels shorter as an initial guess of the PSF.

UNDERPSF = ones(size(PSF) - 4);

[J1,P1] = deconvblind(Blurred, UNDERPSF);

imshow(J1)

| UNDERPSF | blurred |
| --- | --- |

(4) Try OVERPSF, where the uniform array is 4 pixels longer than the true PSF.

OVERPSF = padarray(UNDERPSF, [4 4], 'replicate', 'both');

[J2,P2] = deconvblind(Blurred, OVERPSF);

imshow(J2)

| OVERPSF | blurred |
| --- | --- |

(5) Try the INITPSF whose size is the same as true PSF.

INITPSF = padarray(UNDERPSF,[2 2], 'replicate', 'both');

[J3,P3] = deconvblind(Blurred, INITPSF);

imshow(J3)

| PSF | blurred |
| --- | --- |

It could be observed that PSF has the best solution here compared with UNDERPSF and OVERPSF.

(6) Compared the PSF produced by all above methods.

figure;

subplot(2, 2, 1)

imshow(PSF, [], 'InitialMagnification', 'fit')

title('True PSF')

subplot(222)

imshow(P1, [], 'InitialMagnification', 'fit')

title('Reconstructed Undersized PSF')

subplot(2,2,3)

imshow(P2, [], 'InitialMagnification', 'fit')

title('Reconstructed Oversized PSF')

subplot(2,2,4)

imshow(P3, [], 'InitialMagnification', 'fit')

title('Reconstructed true PSF')

From the image above, we could see that for UNDERPSF, there are fewer value bins generated and thus displaying a far more coarse granularity. In contrary, the OVERPSF guesses a finer granularity which brings in some weird sharpness in its solution. The INITPSF is just a proper approximation of the true PSF, so it deblurs the best.

(7) Try to improve the restoration by adding weight to reduce ringing effect in sharp areas. First try to find sharp pixels using Sobel edge detector.

WEIGHT = edge(Blurred,'sobel', .08);

se = strel('disk',2);

WEIGHT = 1-double(imdilate(WEIGHT,se));

WEIGHT([1:3 end-(0:2)],:) = 0;

WEIGHT(:,[1:3 end-(0:2)]) = 0;

figure; imshow(WEIGHT);

(8) Use the above result as the weight array to do blind deconvolution, so that the sharp edges are less considered during deconvolution.

[J,P] = deconvblind(Blurred,INITPSF,30,[],WEIGHT);

imshow(J)

| deblur on the weight array | blurred |
| --- | --- |

(9) For curiosity, I also took a look at the P, which should be a blur kernel.

imshow(P)

Not surprisingly, it looks like a small black dot, which corresponds to the paper that the size of the blur kernel would be much smaller.

(10) Use additional constraints on PSF.

The deconvblind function accepts a FUN parameter enabling some actions at the end of each iteration, which takes a PSF as argument, and returns a PSF in the same size. Try to use FUN to reduce 2 pixels for OVERPSF to make it similar to INITPSF.